| EOS-3615 | 21CH Mode1 | 16CH Mode2 | DMX Value | Function |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Darkness----->Bright |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---7 8---63 64---71 72---127 128---133 134---189 190--195 196--250 251--255 | Undefined Fast Strobe----->SlowStrobe Undefined Fast Puslation Slow Close--->Fast Puslation Fast Close Undefined Slow Puslation Fast Close--->Fast Puslation Fast Close Undefined Random Strobe Undefined |
| Red | 4 | 3 | 0---255 | Darkness----->Bright |
| Red FINE | 5 |  | 0---255 |  |
| Green | 6 | 4 | 0---255 | Darkness----->Bright |
| Green FINE | 7 |  | 0---255 |  |
| Blue | 8 | 5 | 0---255 | Darkness----->Bright |
| Blue FINE | 9 |  | 0---255 |  |
| White | 10 | 6 | 0---255 | Darkness----->Bright |
| White FINE | 11 |  | 0---255 |  |
| NULL | 12 | 7 | 0---10 11---20 21---30 31---40 41---50 51---60 61---70 71---80 81---90 91---100 101---110 111--199 120---250 251---255 | Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined |
| ZOOM | 13 | 8 | 0---255 |  |
| NULL | 14 | 9 | NULL | NULL |
| Pan | 15 | 10 | 0---255 |  |
| Pan FINE | 16 | 11 | 0---255 |  |
| Tilte | 17 | 12 | 0---255 |  |
| Tilte FINE | 18 | 13 | 0---255 |  |
| NULL | 19 | 14 | NULL |  |
| P/T Speed | 20 | 15 | 0---128 129---170 172---211 212---255 | Fastest Speed----->Slowest Speed Slow Speed(1/ 3 maxspeed) Medium Speed(2/ 3 maxspeed) Fast Speed |
| Control | 21 | 16 | 0---130 131---155 128---255 | Undefined Undefined Reset ALL |