| SEER 1000 Pro-1.01 | 38CH Mode1 | 33CH Mode2 | DMX Value | Function |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Brightness--->Darkness |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---3 4---103 104---107 108---207 208---212 213---225 226---238 239---251 252---255 | Closed Slow Strobe--->Fast Strobe Open SlowPuls---->Fast Puls Open Random Slow Strobe Random Medium Strobe Random Fast Strobe Open |
| Color | 4 | 3 | 0---7 8---14 15----21 22---29 30---36 37---44 45---51 52---59 60---6 67---74 75---81 82---89 90---96 97---104 105---111 112---119 120---126 127---127 128---191 192---255 | White White + Color1 Color1 Color1 + Color2 Color2 Color2 + Color3 Color3 Color3 + Color4 Color4 Color4 + Color5 Color5 Color5 + Color6 Color6 Color6 + Color7 Color7 Color7 + Color8 Color8 Color8 + Color9 CW Rotationg Slow--->Fast CCW Rotationg Fast--->Slow |
| Cyan | 5 | 4 | 0---255 | EXCLUDED---->INSERTED |
| Magenta | 6 | 5 | 0---255 | EXCLUDED---->INSERTED |
| Yellow | 7 | 6 | 0---255 | EXCLUDED---->INSERTED |
| CTC | 8 | 7 | 0---255 | EXCLUDED---->INSERTED |
| Macro Color | 9 |  | 0---255 |  |
| IRIS | 10 | 8 | 0---128 129---131 132---171 172---211 212---251 252---255 | MAX----->MIN Close Slow Puls---->Fast Puls Slow Puls Fast Close---->Fast Puls Fast Close Slow Puls Fast Open---->Fast Puls Fast Open Open |
| Gobo1 Select | 11 | 9 | 0---10 11---20 21---30 31---40 41---50 51---60 61---70 71---80 81---100 101---120 121---140 141---160 161---180 181---200 201---220 221---237 238---255 | White Gobo1 Gobo2 Gobo3 Gobo4 Gobo5 Gobo6 Gobo7 Gobo1 Shake Slow---->Fast Gobo2 Shake Slow---->Fast Gobo3 Shake Slow---->Fast Gobo4 Shake Slow---->Fast Gobo5 Shake Slow---->Fast Gobo6 Shake Slow---->Fast Gobo7 Shake Slow---->Fast CW Rotation Slow--->Fast CCW Rotation Fast---->Slow |
| Gobo1 Index Rotation | 12 | 10 | 0---127 128---191 192---255 | 0°---->360°(FINE IN 13CH) CW Rotating Slow---Fast CCW Rotation Fast---Slow |
| Gobo1 Index Rotation FINE | 13 |  | 0---255 |  |
| Gobo2(Fixed Gobo) | 14 | 11 | 0---6 7---13 14---20 21---27 28---34 35---41 42---48 49---54 55---61 62---68 69---79 80---95 96--- 111 112---127 128---143 144---159 160--- 175 176--- 191 192--- 207 208--- 223 224---239 240---247 248--- 255 | White Gobo1 Gobo2 Gobo3 Gobo4 Gobo5 Gobo6 Gobo7 Gobo8 Gobo9 Gobo10 Gobo1 Shake Slow--->-Fast Gobo2 Shake Slow---->Fast Gobo3 Shake Slow---->Fast Gobo4 Shake Slow---->Fast Gobo5 Shake Slow---->Fast Gobo6 Shake Slow---->Fast Gobo7 Shake Slow---->Fast Gobo8 Shake Slow---->Fast Gobo9 Shake Slow---->Fast Gobo10 Shake Slow---->Fast CW Rotation Slow--->Fast CCW Rotation Fast--->Slow |
| BLADE1 | 15 | 12 | 0---255 | EXCLUDED---->INSERTED |
| BLADE2 | 16 | 13 | 0---255 | EXCLUDED---->INSERTED |
| BLADE3 | 17 | 14 | 0---255 | EXCLUDED---->INSERTED |
| BLADE4 | 18 | 15 | 0---255 | EXCLUDED---->INSERTED |
| BLADE5 | 19 | 16 | 0---255 | EXCLUDED---->INSERTED |
| BLADE6 | 20 | 17 | 0---255 | EXCLUDED---->INSERTED |
| BLADE7 | 21 | 18 | 0---255 | EXCLUDED---->INSERTED |
| BLADE8 | 22 | 19 | 0---255 | EXCLUDED---->INSERTED |
| FRAMING ROTATION | 23 | 20 | 0---255 | Position 0°---->90° |
| Prism | 24 | 21 | 0---127 128---255 | EXCLUDED INSERTED |
| PrismRotation | 25 | 22 | 0---127 128---191 192---255 | Position 0°---->360° CW Rotation Slow-----Fast CCW Rotation Fast-----Slow |
| Forst | 26 | 23 | 0---255 | EXCLUDED------>INSERTED |
| Focus | 27 | 24 | 0---255 |  |
| Focus FINE | 28 |  | 0---255 |  |
| Zoom | 29 | 25 | 0---255 |  |
| Zoom FINE | 30 |  | 0---255 |  |
| Animation | 31 | 26 | 0---5 6---127 128---191 192---255 | White Animation Shake Slow---->Fast CW Rotation Slow----->Fast CCW Rotation Fast----->Slow |
| PAN(X) | 32 | 27 | 0---255 |  |
| PAN(X) FINE | 33 | 28 | 0---255 |  |
| TILTE(Y) | 34 | 29 | 0---255 |  |
| TILTE(Y) FINE | 35 | 30 | 0---255 |  |
| P/T SPEED | 36 | 31 | 0---128 129---170 171---211 212---255 252---255 | Fastest------->Slowest SlowSpeed (1/3 MaxSpeed) MediumSpeed (1/2 MaxSpeed) Fast Speed (Fastest Speed) Darkness when moving |
| Lamp Control | 37 | 32 | 0---32 33---64 65---96 97---128 129---160 161---193 194---225 226---255 | Undefined LAMP OFF( > 3s) Undefined LAMP ON( > 3s) Undefined LOW POWER( > 3s) Undefined FULL POWER( > 3s) |
| Control | 38 | 33 | 0---127 128---153 154---179 180---205 206---231 232---255 | Undefined Undefined Undefined Undefined Undefined Reset ALL( > 3s) |