| Ares-P7II | 39CH Mode1 | 34CH Mode2 | DMX Value | Function |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Brightness--->Darkness |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---3 4---103 104---107 108---207 208---212 213---225 226---238 239---251 252---255 | Closed Slow Strobe--->Fast Strobe Open SlowPuls---->Fast Puls Open Random Slow Strobe Random Medium Strobe Random Fast Strobe Open |
| Color | 4 | 3 |  |  |
| Cyan | 5 | 4 | 0---255 | EXCLUDED---->INSERTED |
| Magenta | 6 | 5 | 0---255 | EXCLUDED---->INSERTED |
| Yellow | 7 | 6 | 0---255 | EXCLUDED---->INSERTED |
| CTC | 8 | 7 | 0---255 | EXCLUDED---->INSERTED |
| Macro Color | 9 |  | 0---255 |  |
| IRIS | 10 | 8 | 0---128 129---131 132---171 172---211 212---251 252---255 | MAX----->MIN Close Slow Puls---->Fast Puls Slow Puls Fast Close---->Fast Puls Fast Close Slow Puls Fast Open---->Fast Puls Fast Open Open |
| Gobo1 Select | 11 | 9 |  |  |
| Gobo1 Index Rotation | 12 | 10 | 0---127 128---191 192---255 | 0°---->360°(FINE IN 13CH) CW Rotating Slow---Fast CCW Rotation Fast---Slow |
| Gobo1 Index Rotation FINE | 13 |  | 0---255 |  |
| Fixed Gobo | 14 | 11 | 0---4 5---10 11---16 17---22 23---28 29---34 35---40 41---46 47---52 53---58 59---74 75---90 91---106 107---122 123---138 139---154 155---170 171--- 186 187--- 202 203---229 230---255 | White Gobo1 Gobo2 Gobo3 Gobo4 Gobo5 Gobo6 Gobo1Shake Slow--->-Fast Gobo2 Shake Slow--->-Fast Gobo3 Shake Slow---->Fast Gobo4 Shake Slow---->Fast Gobo5Shake Slow---->Fast Gobo6 Shake Slow---->Fast Gobo7 Shake Slow---->Fast Gobo8 Shake Slow---->Fast Gobo9 Shake Slow---->Fast CW Rotation Slow--->Fast CCW Rotation Fast--->Slow |
| BLADE1 | 15 | 12 | 0---255 | EXCLUDED---->INSERTED |
| BLADE2 | 16 | 13 | 0---255 | EXCLUDED---->INSERTED |
| BLADE3 | 17 | 14 | 0---255 | EXCLUDED---->INSERTED |
| BLADE4 | 18 | 15 | 0---255 | EXCLUDED---->INSERTED |
| BLADE5 | 19 | 16 | 0---255 | EXCLUDED---->INSERTED |
| BLADE6 | 20 | 17 | 0---255 | EXCLUDED---->INSERTED |
| BLADE7 | 21 | 18 | 0---255 | EXCLUDED---->INSERTED |
| BLADE8 | 22 | 19 | 0---255 | EXCLUDED---->INSERTED |
| FRAMING ROTATION | 23 | 20 | 0---255 | Position 0°---->90° |
| Animation | 24 | 21 | 0---10 11----133 134---255 | White CW Rotation Fast--->Slow CCW Rotation Slow--->Fast |
| Prism1 | 25 | 22 | 0---127 128---255 | EXCLUDED INSERTED |
| PrismRotation1 | 26 | 23 | 0---127 128---190 191---194 195---255 | Position 0°-->360° CW Rotation Fast---> Slow STOP CCW Rotation Slow--->Fast |
| FORST | 27 | 24 | 0---127 128---255 | EXCLUDED INSERTED |
| NULL | 28 | 25 |  |  |
| Focus | 29 | 26 | 0---255 |  |
| Focus FINE | 30 |  | 0---255 |  |
| Zoom | 31 | 27 | 0---255 |  |
| Zoom FINE | 32 |  | 0---255 |  |
| PAN(X) | 33 | 28 | 0---255 |  |
| PAN(X) FINE | 34 | 29 | 0---255 |  |
| TILTE(Y) | 35 | 30 | 0---255 |  |
| TILTE(Y) FINE | 36 | 31 | 0---255 |  |
| P/T SPEED | 37 | 32 | 0---128 129---170 171---211 212---255 | Fastest------->Slowest SlowSpeed (1/3 MaxSpeed) MediumSpeed (1/2 MaxSpeed) Fast Speed (Fastest Speed) |
| NULL | 38 | 33 |  |  |
| Control | 39 | 34 | 0---127 128---153 154---179 180---205 206---231 232---255 | Undefined Undefined Undefined Undefined Undefined Reset ALL( > 3s) |