| LEO-BD30 | 19CH Mode1 | 14CH Mode2 | DMX Value | Function |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Darkness----->Bright |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---7 8---63 64---71 72---127 128---133 134---189 190--195 196--250 251--255 | Undefined Fast Strobe----->SlowStrobe Undefined Fast Puslation Slow Close--->Fast Puslation Fast Close Undefined Slow Puslation Fast Close--->Fast Puslation Fast Close Undefined Random Strobe Undefined |
| Red | 4 | 3 | 0---255 | Darkness----->Bright |
| Red FINE | 5 |  | 0---255 |  |
| Green | 6 | 4 | 0---255 | Darkness----->Bright |
| Green FINE | 7 |  | 0---255 |  |
| Blue | 8 | 5 | 0---255 | Darkness----->Bright |
| Blue FINE | 9 |  | 0---255 |  |
| White | 10 | 6 | 0---255 | Darkness----->Bright |
| White FINE | 11 |  | 0---255 |  |
| Color | 12 | 7 | 0---10 11---20 21---30 31---40 41---50 51---60 61---70 71---80 81---90 91---100 101---110 111--199 120---250 251---255 | Undefined Color1 Color2 Color3 Color4 Color5 Color6 Color7 Color8 Color9 Color10 Undefined Color Loop(Color1<--->Color10) Undefined |
| Focus | 13 | 8 | 0---255 |  |
| Pan(X) | 14 | 9 | 0---255 |  |
| Pan(X) Fine | 15 | 10 | 0---255 |  |
| Tilt(Y) | 16 | 11 | 0---255 |  |
| Tilt(Y) Fine | 17 | 12 | 0---255 |  |
| P/T Speed | 18 | 13 | 0---128 129---170 171---211 212---255 | Fastest------->Slowest SlowSpeed (1/3 MaxSpeed) MediumSpeed (1/2 MaxSpeed) Fast Speed (Fastest Speed) |
| Control | 19 | 14 | 0---127 128---153 154---179 180---205 206---231 232---255 | Undefined Undefined Undefined Undefined Undefined Reset ALL( > 3s) |