| Moore 330 Beam | 22CH Mode1 | 18CH Mode2 | DMX Value | Funciton |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Darkness----->Bright |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---3 4---103 104---107 108---207 208---212 213---225 226---238 239---251 252---255 | Closed Slow Strobe--->Fast Strobe Open Slow Pulsation--->Fast Pulsation Open Random Slow Strobe Random Medium Strobe Random Fast Strobe Open |
| Color | 4 | 3 | 0---4 5---8 9---12 13---17 18---21 22---55 26---29 30--- 34 35--- 38 39---42 43---46 47---51 52---55 56---59 60---63 64---68 69---72 73---76 77---81 82---85 86---89 90---93 94---98 99---102 103---106 107---110 111—115 116---119 120---123 124--- 127 128---191 192---255 | WHITE WHITE + RED RED RED + ORANGE ORANGE ORANGE + AQUAMARI AQUAMARINE AQUAMARINE + GREEN GREEN GREEN + LIGHT GREEN LIGHT GREEN LIGHT GREEN + LAVEN LAVENDER LAVENDER + PINK PINK PINK + YELLOW YELLOW YELLOW + MAGENTA MAGENTA MAGENTA + CYAN CYAN CYAN + CTO 260 CTO 260 CTO 260 + CTO 190 CTO 190 CTO 190 + CTB 8000 CTB 8000 CTB 8000 + BLUE BLUE BLUE + WHITE CW SlowRotation---->Fast Rotation CCW Slow Rotation---->Fast Rotation |
| Gobo1(Fixed Gobo) | 5 | 4 | 0---3 4---7 8---11 12---15 16---19 20---23 24---27 28---31 32---35 36---39 40---43 44---47 48---51 52--- 55 56---59 60---63 64---67 68---71 72---113 114---117 118---159 160---166 167---172 173---179 180---185 186--- 191 192---198 199---204 205---211 212---217 218---223 224---230 231---236 237---243 244---249 250---255 | WHITE GOBO 1 GOBO 2 GOBO 3 GOBO 4 GOBO 5 GOBO 6 GOBO 7 GOBO 8 GOBO 9 GOBO 10 GOBO 11 GOBO 12 GOBO 13 GOBO 14 GOBO 15 GOBO 16 GOBO 17 CW SlowRotation------>FastRotation WHITE CCW SlowRotation------> FastRotation GOBO 2 Shake Slow---->Fast GOBO 3 Shake Slow---->Fast GOBO 4 Shake Slow---->Fast GOBO 5 Shake Slow---->Fast GOBO 6 Shake Slow---->Fast GOBO 7 Shake Slow---->Fast GOBO 8 Shake Slow---->Fast GOBO 9 Shake Slow---->Fast GOBO10 Shake Slow---->Fast GOBO11 Shake Slow---->Fast GOBO12 Shake Slow---->Fast GOBO13 Shake Slow---->Fast GOBO14 Shake Slow---->Fast GOBO15 Shake Slow---->Fast GOBO16 Shake Slow---->Fast |
| Gobo2 Select | 6 | 5 | 0---9 10--- 20 21--- 29 30--- 39 40--- 49 50--- 59 60--- 69 70--- 79 80--- 89 90---104 105---119 120---134 135---149 150---164 165---179 180---194 195---209 210---255 | WHITE GOBO1 GOBO2 GOBO3 GOBO4 GOBO5 GOBO6 GOBO7 GOBO8 GOBO1 Shake Slow---->Fast GOBO2 Shake Slow---->Fast GOBO3 Shake Slow---->Fast GOBO4 Shake Slow---->Fast GOBO5 Shake Slow---->Fast GOBO6 Shake Slow---->Fast GOBO7 Shake Slow---->Fast GOBO8 Shake Slow---->Fast CCW SlowRotation------> FastRotation |
| GOBO2 index Rotation | 7 | 6 | 0---127 128---191 192---255 | Position 0°---->360°(FINE in CH8) CW SlowRotation------> FastRotation CCW SlowRotation------> FastRotation |
| GOBO2 indexRotation FINE | 8 |  | 0---255 | GOBO2 index Rotation FINE |
| PRISM | 9 | 7 | 0---127 128--255 | EXCLUDED INSERTED |
| PRISM Rotation | 10 | 8 | 0---127 128---191 192---255 | Position 0°---->360° CW SlowRotation------> FastRotation CCW SlowRotation------> FastRotation |
| FORST | 11 | 9 | 0---255 | EXCLUDED------>INSERTED |
| FOCUS | 12 | 10 | 0---255 |  |
| FOCUS FINE | 13 |  | 0---255 |  |
| ZOOM | 14 | 11 | 0---255 |  |
| ZOOM FINE | 15 |  | 0---255 |  |
| PAN(X) | 16 | 12 | 0---255 |  |
| PAN(X) FINE | 17 | 13 | 0---255 |  |
| TILTE(Y) | 18 | 14 | 0---255 |  |
| TILTE(Y) FINE | 19 | 15 | 0---255 |  |
| P/T SPEED | 20 | 16 | 0---128 129---170 171---211 212---255 | Fastest------->Slowest Slow Speed ( 1/3 MaxSpeed) Medium Speed ( 1/2 MaxSpeed) Fast Speed (Fastest Speed) |
| Lamp Control | 21 | 17 | 0---32 33---64 65---96 97---128 129---160 161---193 194---225 226---255 | Undefined LAMP ON Undefined LAMP OFF Undefined HALF POWER Undefined FULL POWER |
| Control | 22 | 18 | 0---127 128--255 | Undefined Reset ALL |